# Single crystal adventures: Wombat, the Eulerian cradle, and DMCpy

Siobhan

Contents

My single crystal examples from 2025	1

Kirrily’s clinoatacamite	1

Alex’s mysterious ND crystal	1

Federico’s ginormous Y2SiO5 crystal	1

Software: DMCpy x Wombat	1

Motivation	1

DMCpy	2

DMCpy x Wombat	2

wombatDMCpy	2

wombatDMCpy example using Y2SiO5 data	2

Interactive detector view	2

Reciprocal space view in A-1	2

UB matrix determination	2

Reciprocal space view in r.l.u.	4

# Single crystal examples from 2025

## Kirrily’s clinoatacamite

- This was the first experiment I did using the Eulerian cradle on Wombat

- Was very much a test run and spent some time testing/fixing limits with Frank and Helen

- Did not have an optimal strategy for data collection

  - Initially was doing 0.01° steps which was ridiculous

  - Then did 10° eom scans with 0.1° steps about random echi and epsi angles…

- Sample mounted on Al pin with blu tac (classy)

- Crystal is small and twinned – didn’t cover enough of Q space to find orientation or seriously identify peaks

## Alex’s mysterious ND crystal

- No Eulerian cradle, just sample on Al plate on stick on Wombat sample stage

- Did a long som scan that covered 120° in 0.3° steps

- Smallish crystal is a little bit twinned

- Could probably find orientation of biggest twin?

## Federico’s ginormous Y2SiO5 crystal

fdfjlfkj

# Software: DMCpy x Wombat

## Motivation

- Want a Python script to be able to handle Wombat single crystal data and quickly make projections of data in Qx, Qy, Qz from HDF where steps are of sample rotation angle

- Want to be able to determine rough UB matrix and visualise data in hkl (i.e. r.l.u.)

- Ideally this can handle arbitrary Eulerian cradle angles (!!!)

- Don’t want to use LAMP

- Int3D implementation still a work in progress – this is best for quantitative analysis (ellipsoid integration of peaks) however

  - learning curve for users (and instrument scientists)

  - development of software done by Nebil Kaptcho at ILL so hard to make quick changes

  - need to get ANSTO IT to install it on ANSTO machines

  - runs on its own separate python installation and fortran compiler thing (intimidating)

- Several packages out there however

  - Some have dependencies that only work on Linux e.g. hklpy looks really good https://blueskyproject.io/hklpy/ but ultimately depends on which is impossible to run on Windows (I tried. Other people have also tried. And failed.)

  - Some for x-ray detectors that are point detectors or flat area detectors, or goniometer geometries we don’t have e.g. xrayutilities (Kriegner et al. J. Appl. Cryst. (2013) 46, 1162-1170) https://pypi.org/project/xrayutilities/

  - I thought about which instruments were most similar to Wombat which led me to …

## DMCpy

- DMC is a cold neutron diffractometer at PSI that has an area detector very similar to Wombat (in fact Wombat’s new detector will be like DMC’s)

- DMCpy is a python package for reduction of DMC single crystal and powder data developed at DMC by Jakob Lass, Samuel Harrison Moody, and Øystein Slagtern Fjellvåga

- It pretty much does everything we need (except we do powder reduction differently)

- It is available from pip https://pypi.org/project/DMCpy/

- It is described in this pre print https://arxiv.org/abs/2501.08845

- Documentation is here https://dmcpy.readthedocs.io/en/latest/index.html including some tutorials (scripts only, no example data)

- Public Git repo is https://github.com/Jakob-Lass/DMCpy

# DMCpy x Wombat

## wombatDMCpy

To avoid ambiguity, I changed the name of my local copy of DMCpy to wombatDMCpy. To make DMCpy work for Wombat, the big changes were:

- Different HDF file structure (hence new WombatFileStructure)

- Different way of organising histogram data

- Different definitions of some angles etc

  - A3 is DMCpy’s som

  - -A4 is their 2θ

  - Z is their vertical detector position (angle between sample and vertical detector position is called alpha)

- Different detector geometry and sample rotation options (hence new classes WombatDataFile and WombatDataSet)

  - Wombat can rotate the sample about som, echi, epsi, eom, and the sample environment stick axis (forgot what this is called)

Some small tweaks to the functionality of Interactive Viewer (raw detector view) and Viewer 3D (reciprocal space view) have also been made for the Wombat context.

NB: no work has been done to make the powder diffraction functions of DMCpy compatible with Wombat

wombatDMCpy is available at https://github.com/queenofthefairies/Wombat_DMCpy/tree/wombat_WIP (Wombat_WIP branch as original DMCpy package is on master branch currently)

I have been running wombatDMCpy in its own venv in MS Visual Studio code. It should also work in Spyder etc but this might kill the interactivity in the plot windows.

## wombatDMCpy example using Y2SiO5 data

Scripts and test data are available here: https://github.com/queenofthefairies/Wombat_DMCpy/tree/wombat_WIP/wombat_Y2SiO5_example

### Interactive detector view

Here you can look at the raw data, step by step (by moving the scroll bar, in this example, steps in ephi), as well as integrating over the detector vertically (integrating over z) or horizontally (integrating over 2θ). All colour scale limits can be set in the script.

Script: Detector_View_Y2SiO5.py

### Reciprocal space view in A-1

Qx-Qy is the scattering plane of the detector. Qz is the out-of-plane component.

If merging HDF together, the HDF files must have the same number of steps!

Script: Reciprocal_Space_View_invAA_Y2SiO5.py

### UB matrix determination

Before calculating a UB matrix, you need to know the unit cell, and it is necessary to index some peaks by hand.

This is what I did:

- Used the reciprocal space viewer (Qx, Qy, Qz in inv Angstroms) script: Reciprocal_Space_View_invAA_Y2SiO5.py

- Chose a couple of peaks from different planes and noted their coordinates

- Used the reciprocal space conversion spreadsheet to figure out d spacing, lined up d spacings or 2θ with allowed peaks!

- Noted the Qx, Qy, Qz and corresponding hypothesised h, k, l

- Put these into the script

#### Worked example:

Zoom is helpful here!

Co-ords of this peak (Qx, Qy, Qz) =

(-0.064, 2.145, -0.005)

Co-ords of this other peak (Qx, Qy, Qz) =

(-3.972, -1.765, -0.1549)

Peaks are identified as (2, 0, 2) and (4, 0, 4). Not a good idea to use two peaks on the same line so chose (2, 0, 2) and (4, 0, -2).

Put some other peaks in there too, although UB calculation can only use two peaks + unit cell as input

## Reciprocal space view in r.l.u.